| Assessment Number: | PRA 003 |  |  | Assessment Date: |  |  |  |  | Legislation/Codes of Practice Work Health and Safety Act 2011 Work Health and Safety Regulations 2011 Environmental Protection Act 1994 Electrical Safety Regulations 2013 Managing Risks of Plant in the Workplace 2013 Managing Noise and Preventing Hearing Loss at Work Code of Practice 2011 Managing the Risk of Falls At Workplaces Code of Practice 2011 Managing Risks of Hazardous Chemicals in the Workplace COP 2013 Hazardous Manual Tasks Code of Practice 2011 Excavation Works Code of Practice 2013 Working near Overhead and Underground Electric lines Code of Practice 2010 Working near Overhead and Underground Electric lines Code of Practice 2010 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Plant Type: | Tipper Truck and Trailer |  |  |  |  |  |  |  |  |
| Plant Make: |  |  |  | Plant Model: |  |  |  |  |  |
| Asset/Fleet/Rego No: |  |  |  | Plant Serial No. |  |  |  |  |  |
| Make / Type of Quick Hitch Device (if fitted) |  |  |  |  | Quick Hitch Device Serial No. |  |  |  |  |
| Plant Owner Name: |  |  |  |  |  |  |  |  |  |
| Assessment Facilitated by: |  | Leigh Crawford |  |  |  |  |  |  |  |
| Signature: |  |  |  |  |  | Date: |  |  |  |
| Assessment Participants: |  |  |  |  |  |  |  |  |  |
| Initial Assessment Follow up Assessment (See below) |  |  |  |  |  |  |  |  |  |
| Follow up based on change to: Use of plant System of work Plant Environment New or additional information Plant through modification |  |  |  |  |  |  |  |  |  |

Any hazard assessed as presenting a low and/or medium risk level will be controlled using a combination of controls as appropriate.

Any hazard assessed as presenting a high risk level must be controlled using a combination of at least one engineering control and lower level controls as appropriate.  Where this is not possible, Workplace Manager consultation will take place.

Any hazard assessed as presenting an extreme risk level will be controlled using elimination and engineering as the primary source of controls. Where this is not possible, Workplace Manager consultation will take place.

| Is the plant designed to perform the task? | Yes | No |  |  |
| --- | --- | --- | --- | --- |
| Has the plant been modified from the original condition? | Yes | No |  |  |
| Is the plant in good working condition and free of weeds & mud? | Yes | No |  |  |
| All identified action items closed out/addressed (plant checks)? | Yes | No |  |  |
| Is the plant safe to operate? (On completion of PHA and action closure) | Yes | No | Date: | Signature: |

Risk Management Process

- The likelihood of an incident occurring as a result of the hazard will first be assessed.

| Likelihood Ratings |  |  |
| --- | --- | --- |
| A | Almost Certain | The event is expected to occur in most circumstances |
| B | Likely | The event will probably occur in most circumstances |
| C | Possible | The event may occur at some time |
| D | Unlikely | The event could occur at some time |
| E | Rare | The event may only occur in exceptional circumstances |

- The consequences (if an incident did occur) will then be determined. To determine the possible consequences, a judgement on the severity of the potential outcome will be made.

| Consequence Ratings |  |  |
| --- | --- | --- |
| 1 | Insignificant | Nil injuries |
| 2 | Minor | First aid treatment, on-site release immediately contained |
| 3 | Moderate | Medical treatment, on-site release contained with outside assistance |
| 4 | Major | Extensive injuries, loss of capability, off-site release with no detrimental affects |
| 5 | Catastrophic | Death, release off-site with detrimental effect |

- The likelihood and consequences estimates will then be combined to obtain a total risk score by using the following risk priority table.

- The following legend will be used to determine the response.

| Legend |  |  |
| --- | --- | --- |
| Score |  | Action |
| E | 18 – 25 | Extreme Risk. Requires immediate attention |
| H | 10 – 17 | High Risk. Senior management attention required urgently |
| M | 6 – 9 | Moderate Risk. Follow management instructions and procedures |
| L | 1 - 5 | Low Risk. Record and review if processes change. Monitor |

- Control measures will be implemented using the following hierarchy of controls.

| Hazard identification and risk assessment during operations and / or maintenance activities |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section 1 Put an X if the hazard does apply to the plant. Leave blank if the hazard does not apply to the plant |  |  |  |  |  |  |  |  |  | Section 4 Then indicate the Likelihood, Consequence and Risk Rating |  |  |  |
| Section 2 Write where and when the plant hazard may exist |  |  |  |  |  |  |  |  |  | Section 5 Write the existing Controls and relevant Comments relating to additional controls required |  |  |  |
| Section 3 Indicate when the exposure is likely to occur? During Operations (O), Maintenance (M) or Both (B) |  |  |  |  |  |  |  |  |  | Section 6 Indicate the residual risk taking into account controls being implemented after considering applicable legislation, Codes, Standards, etc |  |  |  |
| SECTION 1 Hazard category and examples |  | SECTION 2 Where and When may this hazard exist? |  | SECTION 3 Exposure during O M or B? | SECTION 4 Risk |  |  |  | SECTION 5 Controls and comments |  | SECTION 6 Residual Risk |  |  |
|  |  |  |  |  | L |  | C | R |  |  | L | C | R |
| Entanglement - Yes No Can anyone’s hair, clothing, gloves, necktie, jewellery, rags and other materials become entangled with moving parts of plant, or materials in motion? |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Arms, hands, fingers, or upper body Legs, feet, or lower body Hair, clothing or jewellery Isolation of energy source Other (please specify) ___________________ | - During normal operation - During routine maintenance - Around moving or rolling parts - Around front of plant - Mechanical moving parts during pre-start inspections |  | B |  |  | B | 3 | H 17 | Plant Controls - Seat Belt fitted and to be worn at all times - Guards secure and in place - Reversing alarm - Rotating Beacon - Horn - Parking Break – Reverse park truck - UHF Radio - Safety decals as per manufacturer’s specifications Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Ensure hands, fingers, loose items of clothing and jewellery are kept away from moving parts - Operate truck in accordance with the manufacturers operating and maintenance manual - Only qualified and competent personnel to complete repair works |  | E | 3 | M 6 |

| Crushing - Yes No Can anyone be crushed due to falling, uncontrolled or unexpected movement of plant or its load, lack of capacity to slow, stop or immobilise the plant, tipping or rolling over, parts of plant collapsing, contact with moving parts during testing, inspection, maintenance, cleaning or repair, thrown off, under or trapped between plant and materials or fixed structures? |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Exposed edges (falling) Tools being dropped Material falling or being ejected from working area Moving components Plant overturning Uncontrolled movement of loads Other (please specify) ___________________ | - All around plant - Access doors - Loading and Unloading plant - During normal operation - During routine maintenance | B | B | 3 | H 17 | Plant Controls - Seat Belt fitted and to be worn at all times - Reversing alarm - Rotating Beacon - Horn - Parking Break – Reverse park truck - UHF Radio - Safety decals as per manufacturer’s specifications Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Ensure seat belt is used when driving truck - Operate truck in accordance with the manufacturers operating and maintenance manual - Awareness of other workers in vicinity of work area and ensure EW is utilised and remains in contact with the driver via UHF. EWs must be trained in safe work procedures whilst working around operational truck - Awareness of hazards as per daily toolbox. Pre-start checklist to be completed daily prior to commencing work - Truck movement plans for work areas discussed at pre-start toolbox - Maintain a 15m exclusion zone when tipping. EW to be in radio and eye sight contact at all times. Stop if EW cannot be seen, contact EW on UHF and only resume operation EW comes back into your line of sight - Do not over load truck - Secure all loads - Remove all persons from tipping area. Check area before tipping load - Only qualified and competent personnel to complete repair works - No maintenance work to be conducted under a raised tipper body until security bars are safely locked into position | E | 3 | M 6 |

| Cutting/ Stabbing/ Puncturing - Yes No Can anyone be cut, stabbed or punctured by coming in contact with moving plant or parts, sharp or flying objects, work pieces ejected, work pieces disintegrated or other factors not mentioned? |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact with flying parts or work pieces Parts or work pieces breaking (disintegrating) Work pieces ejected Movement of plant or components Isolation of energy sources Body or body parts caught between moving components Other (please specify) ___________________ | - All around plant - During normal operation - During routine maintenance - Use of attachments: rock breaker, ripper, post hole borer | B | B | 3 | H 17 | Plant Controls - Seat Belt fitted and to be worn at all times - Guards secure and in place - Reversing alarm - Rotating Beacon - Horn - Parking Break – Reverse park truck - UHF Radio - Safety decals as per manufacturer’s specifications Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Ensure seat belt is used when driving truck - Operate truck in accordance with the manufacturers operating and maintenance manual - Awareness of other workers in vicinity of work area and ensure EW is utilised and remains in contact with the driver via UHF. EWs must be trained in safe work procedures whilst working around operational truck - Awareness of hazards as per daily toolbox. Pre-start checklist to be completed daily prior to commencing work - UHF communication, line of sight maintained and EW in place at all times - Only qualified and competent personnel to complete repair works | E | 3 | M 6 |

| Shearing - Yes No Can anyone’s body parts be cut off between two parts of the plant and a work piece or structure? |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Body or body parts caught between moving components Isolation of energy sources Body or body parts shear when passing structure Other (please specify) ___________________ | - All around plant - As plant passes any structure - During normal operation - During routine maintenance | B | B | 3 | H 17 | Plant Controls - Seat Belt fitted and to be worn at all times - Guards secure and in place - Reversing alarm - Rotating Beacon - Horn - Parking Break – Reverse park truck - UHF Radio - Safety decals as per manufacturer’s specifications Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Ensure seat belt is used when driving truck - Awareness of other workers in vicinity of work area and ensure EW is utilised and remains in contact with the driver via UHF. EWs must be trained in safe work procedures whilst working around operational truck - Awareness of hazards as per daily toolbox. Pre-start checklist to be completed daily prior to commencing work - Operate truck in accordance with the manufacturers operating and maintenance manual - UHF communication, line of sight maintained and EW in place at all times - Ensure hands, fingers, loose items of clothing and jewellery are kept away from moving parts - Only qualified and competent personnel to complete repair works | E | 3 | M 6 |

| Striking - Yes No Can anyone be struck by moving objects due to plant or surfaces of the plant, or material handled by plant operation? |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uncontrolled or unexpected movement of plant (warning sirens required) Uncontrolled or unexpected movement of components or material (warning sirens required) Moving objects due to Parts or work pieces breaking (disintegrating) Work materials protruding into travel path of Plant. Normal movement of plant Isolation of energy sources Other (please specify) ___________________ | - All around plant - During normal operation - During routine maintenance | B | B | 3 | H 17 | Plant Controls - Seat Belt fitted and to be worn at all times - Guards secure and in place - Reversing alarm - Rotating Beacon - Horn - Parking Break – Reverse park truck - UHF Radio - Safety decals as per manufacturer’s specifications Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - UHF communication, line of sight maintained and EW in place at all times - Truck stand down procedure: lower tipper body, switch off truck and remove key prior to exiting. Do not restart truck until all EWs are safe behind barrier and received “go ahead” radio communication - Awareness of other workers in vicinity of work area and ensure EW is utilised and remains in contact with the driver via UHF. EWs must be trained in safe work procedures whilst working around operational truck - Ensure seat belt is used when driving truck - Ensure all access points are clear of obstacles - Operate truck in accordance with the manufacturers operating and maintenance manual - Ensure all onsite/road rules, safety signs and speed limits are abided to. No overloading - Check your load before driving off making sure that there is nothing sticking out of the sides of the tipper. - Check your bin after you tip each load to ensure that there is no spoil left over - Lower tipper body before exiting the tipping area - No loose / unsecure items ie: toolbox, fire extinguisher, shovel, etc… All loose items need to be secured at all times - Ensure that tailgate is closed and secure at all times (when not in tipping mode) - Only qualified and competent personnel to complete repair works | E | 3 | M 6 |

| Electricity (Shock or burns) Contact - Yes No Can anyone be injured by electrical shock or burnt due to damaged or poorly maintained leads or switches, water near electrical equipment, working near or contact with live electrical conductors, lack of isolation procedures or the factors not mentioned? |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact via damaged or poorly maintained electrical leads and cables Overloading of electrical circuits Isolation of electrical energy sources Contact with or proximity to live electrical conductors Contact via damaged electrical control devices Contact via water entry Other (please specify) ____________________ | - Loading and Unloading Plant - During normal operation - During routine maintenance | B | B | 3 | H 17 | Plant Controls - Truck Isolation - Tag out procedure to be implemented in the event of the truck coming into contact with power lines. This will prevent the use of the truck whilst locked out. Driver or fitter/maintenance person to place an “out of service” tag on the truck. Truck to be locked and keys removed and handed to fitter/maintenance person. Once the truck is again suited for operation the person that placed the tag will remove it. Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Only qualified and competent personnel to complete repair works - Any aboveground services to the identified, clearly marked and tool boxed - Ensure the regulatory safe distances are adhered to when working around power lines.(maintain 3m distance up to 132kv and a minimum 8m over 132kv) - Ensure EW is utilised and remains in contact with the driver via UHF when working around services. EWs must be trained in safe work procedures whilst working around operational truck - Ensure to look up and check for power lines or any other obstacle prior to raising the tipper body - In the event the truck comes into contact with power lines, the driver is to remain in the truck. If possible lower the tipper body and drive away from the fallen power lines. Immediately contact EW via UHF to advise of incident allowing the EW to further restrict access to the area. Site supervisor to be advised of incident immediately. Driver to remain in the truck until given the all clear by the relevant authorities. truck to be tagged out until inspected by a qualified competent person | E | 3 | M 6 |

| Explosion/Fire - Yes No Can anyone be injured by an explosion of gas, vapours, liquids, dusts or other substances, triggered by plant operation? |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ignition of flammable atmosphere initiated by the plant Ignition of flammable atmosphere initiated by material Ignition of flammable material by the plant Ignition of flammable material by the process Other (please specify) ____________________ | - During normal operation - During routine maintenance | B | C | 2 | M 8 | Plant Controls - Fire Extinguisher fitted to truck. Current test and tag (within 6 months) - Safety decals as per manufacturer’s specifications Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Switch off truck when refuelling – Refer to Refuelling SWMS - Truck not to be operated near flammable substances (no smoking, no naked flames) - Only qualified and competent personnel to complete repair works | D | 2 | L 5 |
| Slips/ Trips/ Falls - Yes No Can anyone using the plant or in the vicinity of the plant, slip, trip or fall due to the working environment or other factors? |  |  |  |  |  |  |  |  |  |
| Uneven or slippery work or access surfaces entering or exiting the plant Housekeeping hazards produced by the plant Inadequate work platforms (size, location, fall protection) Lack of guardrails or fall protection Other (please specify) ____________________ | - During normal operation - During routine maintenance - Front access into plant - Accessing and exiting plant - Pre-start inspections | B | C | 3 | H 13 | Plant Controls - Steps and handrails are clean and secure - Safety decals as per manufacturer’s specifications Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Maintain three points of contact when entering and exiting truck - Ensure appropriate foot wear is used - Clean up all oil, fuel and grease spills around access area - Do not attempt to access the back of the tipping body or load - Truck must be fitted with retractable tarps able to be operated from ground level | E | 3 | M 6 |

| High Pressure Fluid - Yes No Can anyone come into contact with fluids under high pressure, due to failure or misuse of the plant? |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact with fluids or gas under pressure due to failure Striking due to severed high pressure hoses/couplings Isolation and bleeding of pressure energy sources Other (please specify) ____________________ | - During normal operation - During routine maintenance | B | C | 3 | H 13 | Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure plant is operated by trained, qualified, competent and authorised personnel - Correct PPE to be worn – Refer to applicable SWMS or perform a risk assessment - Only trained, qualified, competent and authorised personnel to complete repair works - De-energise/De-pressurise hydraulic system prior to carrying out maintenance - Cool down/De-pressurise cooling system prior to carrying out maintenance - Inspect hydraulic lines. Damaged lines and connectors cannot be re-used | E | 3 | M 6 |
| Plant rolling over/ through limits - Yes No Can this item of plant roll or tip over due to operating over specified working limits? |  |  |  |  |  |  |  |  |  |
| Tip over hazards. Upper and lower limits Other (please specify) ____________________ | - During normal operation - Travelling inclines - Roll or tip over when extending boom to lower pipes | B | B | 3 | H 17 | Plant Controls - Seat Belt fitted and to be worn at all times Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Ensure truck is operated on level ground - Operate truck in accordance with the manufacturers operating and maintenance manual - Ensure all onsite/road rules, safety signs and speed limits are abided to at all times - Do not over load truck - Check that ground condition is suitable for the operation of truck ie: do not drive truck on mud/boggy terrain - Ensure that you tip in the allocated tipping area. You must tip on flat level ground - Keep a minimum of 5m distance from exposed trenches or other similar hazards | E | 3 | M 6 |

| Working environment and ergonomics - Yes No Can anyone be injured due to seating design, repetitive body movement or posture, excessive effort, poor workplace or plant design causing mental or physical stress, lack of consideration for human behaviour, poor lighting or others factors not mentioned? |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inadequate lighting levels Glare from artificial light Glare from natural light Placement and identification of controls Seating design or seating location Human error or behaviour aspects (Human factors) Manual handling tasks associated with the plant Cramped or restricted work spaces (particularly for maintenance Noise levels Vibration Rain or moisture Radiation (ionising – non ionising) Biological Heat and UV radiation Location of plant in the workplace Other (please specify) ____________________ | - During normal operation - During routine maintenance | B | B | 2 | H 12 | Plant Controls - Seat Belt fitted and to be worn at all times - Manufacturer approved driver’s seat - Fire Extinguisher fitted to truck. Current test and tag (within 6 months) - Controls clearly marked - Safety decals as per manufacturer’s specifications Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Use portable lighting towers if inadequate lighting levels - Maintain three points of contact when entering and exiting truck - Ensure truck is not operated in confined spaces, allowing constant airflow - If issues occur ensure that these are placed on daily report and advised to the supervisor - Apply sunscreen regularly to exposed skin - Drink water regularly to maintain hydration - Driver to take regular breaks as required - Driver must not place mud or debris on road way | D | 2 | L 5 |

| Other Hazards – Yes No Can anyone be injured or suffer ill health from exposure to: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chemicals Toxic Gases Vapours Fumes Other (please specify) ____________________ | - During normal operation - During routine maintenance | B | C | 3 | H 13 | Plant Controls - Fire extinguisher fitted to truck. Current test and tag (within 6 months) - Truck air conditioning unit to be serviced in accordance with manufacturer’s specifications Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Drivers handling chemicals must be trained in SDS and hazards associated with substances - SDS’s available - Drivers inducted into site emergency procedures - Switch off truck when refuelling – Refer to Refuelling SWMS - Ensure truck is not operated in confined spaces, allowing constant airflow | E | 3 | M 6 |
| Condition and suitability of plant |  |  |  |  |  |  |  |  |  |
| Age and condition Service and maintenance history Frequency of use (high or low use or inappropriate duty cycle) Not fit for purpose Unsuitable accessories/fittings Inability to apply isolation/lock out devices Modification from original design Other (please specify) ____________________ | - During normal operation - During routine maintenance | B | C | 3 | H 13 | Plant Controls - Truck serviced in accordance with manufacturer’s specifications - Operators manual available in plant at all times Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure plant is operated by trained, qualified, competent and authorised personnel - Correct PPE to be worn – Refer to applicable SWMS or perform a risk assessment - Only trained, qualified, competent and authorised personnel to complete repair works | E | 3 | M 6 |

| System of work relating to the plant |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emergency procedures relating to the plant Communication systems associated with plant operation Communication methods with plant operation Use of Permit to Work system Start up and shut down procedures Secure against unauthorised use/access Storage or restoration to service requirements Other (please specify) ____________________ | - During normal operation - During routine maintenance | B | C | 3 | H 13 | Plant Controls - Seat Belt fitted and to be worn at all times - Reversing alarm - First Aid Kit located inside the truck’s cabin - Rotating Beacon - Fire Extinguisher fitted to truck. Current test and tag (within 6 months) - UHF Radio - Parking Break – Reverse park truck - Safety decals as per manufacturer’s specifications Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure truck is operated by competent licenced person - Correct PPE to be worn – Steel cap boots, Hi Vis long sleeve shirt, Long pants, Hard hat with brim, Gloves, Ear / Eye protection - Emergency equipment maintained ie: fire extinguisher test and tagged within 6 months - Ensure all drivers are inducted into site emergency procedures - Activate site emergency procedures in the event of an emergency - Driver to ensure that keys are removed and truck is locked when away from the vehicle - Activate parking brake before leaving cabin - At the end of the shift driver must reverse park the truck, switch off truck and remove keys. If truck has a turbo timer installed then the driver must remain near the truck until its switched off | E | 3 | M 6 |

| Environmental issues causes failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inclement weather causes issues Wind fowls cables and snags or breaks cable Water impairs operation Wind speed exceeds recommended limit Other (please specify) ____________________ | - During normal operation | B | C | 3 | H 13 | Operator / Procedural Controls - Complete daily prestart checklist prior to commencing work - Ensure plant is operated by trained, qualified, competent and authorised personnel - Correct PPE to be worn – Refer to applicable SWMS or perform a risk assessment - Check ground conditions prior to operating the plant - Ensure not to contaminate roads, kerb and gutters - In the event of a spill operator to cease all works immediately, lower boom, switch off plant and advise site supervisor immediately. With the assistance of Essential Worker, operator to grab a spill kit from the site office and clean up the spill in accordance with Principal’s site procedures | E | 3 | M 6 |

COMMENTS:

| Are all identified risks controlled to as low as reasonably practicable? |  | YES: or NO: If No, list Additional controls required on next page |  |
| --- | --- | --- | --- |
| Completed by: | Leigh Crawford | Contact details: | 0413 337 873 |
| Reviewed by: |  | Contact details: |  |

I have reviewed the Plant Hazard and Risk Assessment and have had the opportunity to comment and make changes as I thought necessary.

| Name: | Position: | Signature: | Date: | Plant Model No: | Plant Serial No: | Company: |
| --- | --- | --- | --- | --- | --- | --- |

Additional controls: For each additional control, identify appropriate corrective actions, including priority, timeframes and responsibilities, communicate the requirements to the person responsible and then input the information into the Corrective Action Register.